High Performance Computing Lab

Practical No. 10

Title of practical: Understanding concepts of CUDA Programming

Problem Statement 1:

Execute the following program and check the properties of your GPGPU.

#include <stdio.h>

#include <stdlib.h>

int main()

{

int deviceCount;

cudaGetDeviceCount(&deviceCount);

if (deviceCount == 0)

{

printf("There is no device supporting CUDA\n");

}

int dev;

for (dev = 0; dev < deviceCount; ++dev)

{

cudaDeviceProp deviceProp;

cudaGetDeviceProperties(&deviceProp, dev);

if (dev == 0)

{

if (deviceProp.major < 1)

{

printf("There is no device supporting CUDA.\n");

}

else if (deviceCount == 1)

{

printf("There is 1 device supporting CUDA\n");

}

else

{

printf("There are %d devices supporting CUDA\n", deviceCount);

}

}

printf("\nDevice %d: \"%s\"\n", dev, deviceProp.name);

printf("  Major revision number:                         %d\n", deviceProp.major);

printf("  Minor revision number:                         %d\n", deviceProp.minor);

printf("  Total amount of global memory:                 %d bytes\n", deviceProp.totalGlobalMem);

printf("  Total amount of constant memory:               %d bytes\n", deviceProp.totalConstMem);

printf("  Total amount of shared memory per block:       %d bytes\n", deviceProp.sharedMemPerBlock);

printf("  Total number of registers available per block: %d\n", deviceProp.regsPerBlock);

printf("  Warp size:                                     %d\n", deviceProp.warpSize);

printf("  Multiprocessor count:                          %d\n",deviceProp.multiProcessorCount );

printf("  Maximum number of threads per block:           %d\n", deviceProp.maxThreadsPerBlock);

printf("  Maximum sizes of each dimension of a block:    %d x %d x %d\n", deviceProp.maxThreadsDim[0],deviceProp.maxThreadsDim[1], deviceProp.maxThreadsDim[2]);

printf("  Maximum sizes of each dimension of a grid:     %d x %d x %d\n", deviceProp.maxGridSize[0], deviceProp.maxGridSize[1],  deviceProp.maxGridSize[2]);

printf("  Maximum memory pitch:                          %d bytes\n", deviceProp.memPitch);

printf("  Texture alignment:                             %d bytes\n", deviceProp.textureAlignment);

printf("  Clock rate:                                    %d kilohertz\n", deviceProp.clockRate);

}

}

Problem Statement 2:

Write a program to where each thread prints its thread ID along with hello world. Lauch the kernel with one block and multiple threads.

Problem Statement 3:

Write a program to where each thread prints its thread ID along with hello world. Lauch the kernel with multiple blocks and multiple threads.

Problem Statement 4:

Write a program to where each thread prints its thread ID along with hello world. Lauch the kernel with 2D blocks and 2D threads.

Problem statement 5: Vector Addition using CUDA

Problem Statement: Write a CUDA C program that performs element-wise addition of two vectors A and B of size N. The result of the addition should be stored in vector C.

Details:

•	Initialize the vectors A and B with random numbers.

•	The output vector C[i] = A[i] + B[i], where i ranges from 0 to N-1.

•	Use CUDA kernels to perform the computation in parallel.

•	Write the code for both serial (CPU-based) and parallel (CUDA-based) implementations.

•	Measure the execution time of both the serial and CUDA implementations for different values of N (e.g., N = 10^5, 10^6, 10^7).

Task:

•	Calculate and report the speedup (i.e., the ratio of CPU execution time to GPU execution time).

________________________________________

Problem statement 6: Matrix Addition using CUDA

Problem Statement: Write a CUDA C program to perform element-wise addition of two matrices A and B of size M x N. The result of the addition should be stored in matrix C.

Details:

•	Initialize the matrices A and B with random values.

•	The output matrix C[i][j] = A[i][j] + B[i][j] where i ranges from 0 to M-1 and j ranges from 0 to N-1.

•	Write code for both serial (CPU-based) and parallel (CUDA-based) implementations.

•	Measure the execution time of both implementations for various matrix sizes (e.g., 100x100, 500x500, 1000x1000).

Task:

•	Calculate the speedup using the execution times of the CPU and GPU implementations.

________________________________________

Problem statement 7: Dot Product of Two Vectors using CUDA

Problem Statement: Write a CUDA C program to compute the dot product of two vectors A and B of size N. The dot product is defined as:

Details:

•	Initialize the vectors A and B with random values.

•	Implement the dot product calculation using both serial (CPU) and parallel (CUDA) approaches.

•	Measure the execution time for both implementations with different vector sizes (e.g., N = 10^5, 10^6, 10^7).

•	Use atomic operations or shared memory reduction in the CUDA kernel to compute the final sum.

Task:

•	Calculate and report the speedup for different vector sizes.

______________________________________ __

Problem statement 8: Matrix Multiplication using CUDA

Problem Statement: Write a CUDA C program to perform matrix multiplication. Given two matrices A (MxN) and B (NxP), compute the resulting matrix C (MxP) where:

Details:

•	Initialize the matrices A and B with random values.

•	Write code for both serial (CPU-based) and parallel (CUDA-based) implementations.

•	Measure the execution time of both implementations for various matrix sizes (e.g., 100x100, 500x500, 1000x1000).

Task:

•	Calculate the speedup by comparing the CPU and GPU execution times.